3D LiDAR SCANNER PROJECT

Project Progress Handout & System Summary

# 1. Project Overview

The project is a low-cost 3D LiDAR scanning system that converts a 2D YDLIDAR X2 into a 3D mapping platform. A Raspberry Pi 5 acts as the main computing unit, while an ESP32 controls the rotating platform through a DRV8825 stepper-driver interface. The system captures 2D LiDAR scans at different platform angles and combines them into a 3D point cloud for real-time visualization.

The current implementation is designed as an embedded, portable and expandable platform. The software is intended to support future AI-based processing and applications such as object detection, navigation and digital mapping.

# 2. Main Hardware

| Component | Role | Current Configuration |
| --- | --- | --- |
| Raspberry Pi 5 | Main computer and scanner host | Ubuntu 24.04 LTS |
| YDLIDAR X2 | 2D distance/laser scanning | USB serial connection |
| ESP32 | Motor controller and IMU interface | 115200 baud serial |
| DRV8825 | Stepper motor driver | STEP/DIR control |
| NEMA17 stepper motor | Rotates the LiDAR platform | Mechanical rotation |
| MPU-6050 | Physical motion/rotation sensing | I²C via ESP32 |

# 3. Software Stack

| Software / Library | Purpose | Status / Use |
| --- | --- | --- |
| Ubuntu 24.04 LTS | Operating system on Raspberry Pi | Current platform |
| Python 3.11 | Main scanner application | Current scanner runtime |
| YDLIDAR SDK | LiDAR data acquisition | Connected to X2 |
| NumPy | Numerical and point-cloud calculations | Core processing |
| PySerial | Raspberry Pi ↔ ESP32 communication | Motor/control protocol |
| PyQt6 | GUI framework | Scanner interface |
| VisPy / OpenGL | GPU-accelerated 3D rendering | Interactive point cloud |
| Arduino / AccelStepper | ESP32 firmware and stepper control | Motor firmware |

# 4. How the System Works

1. The YDLIDAR X2 performs a 360° 2D scan and provides distance/angle measurements to the Raspberry Pi.

2. The Raspberry Pi runs the Python scanner application and collects the LiDAR data.

3. The Raspberry Pi communicates with the ESP32 through USB serial communication.

4. The ESP32 controls the NEMA17 stepper motor through the DRV8825 driver and rotates the LiDAR platform.

5. After a platform movement, the ESP32 reports the platform angle using the existing ANGLE/DONE protocol.

6. The MPU-6050 provides an additional physical-motion signal so the ESP32 can check whether rotation was actually detected.

7. The Raspberry Pi associates LiDAR points with the platform angle and converts the measurements into 3D coordinates.

8. VisPy/OpenGL displays the resulting point cloud interactively with zoom, pan and camera rotation.

# 5. Current ESP32 Motor Protocol

The original working ESP32 protocol has been preserved. This is important because the Raspberry Pi scanner already uses these messages for synchronization.

NEXT — requests one platform increment.

MOVE:<angle> — requests a relative platform rotation.

HOME — resets the logical platform angle to 0°; the current firmware does not use a physical limit switch.

SETSTEP:<value> — acknowledged for compatibility with the Raspberry Pi software.

MOVING — indicates that a requested movement has started.

ANGLE:<value> — reports the logical LiDAR platform angle.

DONE — indicates that the commanded movement has completed.

ROTATED / ROTATION_NOT_DETECTED — additional IMU-based physical-movement status.

# 6. Angular Movement Update

The previous configuration used a 0.6° platform increment per NEXT command. The current testing configuration is being changed to a 1.5° platform increment to make the individual platform movement easier for the MPU-6050 to detect.

At 1.5° per increment, a full 360° platform rotation requires 240 positions. This is a larger step between slices than 0.6°, so it improves movement visibility and reduces the number of slices, while 0.6° provides finer angular sampling. The 1.5° setting is therefore primarily being used for testing and synchronization validation at this stage.

# 7. MPU-6050 Integration Progress

The MPU-6050 is connected to the ESP32 through I²C using SDA and SCL.

Startup gyro calibration averages stationary samples to estimate gyro bias.

The corrected gyro signal is monitored during each motor movement.

The ESP32 records the maximum observed gyro Z magnitude and accumulated gyro activity during the movement.

The IMU is being used as a physical-movement verification signal rather than as the primary LiDAR platform-angle source.

The stepper's commanded/logical angle remains the main angular reference for 3D reconstruction because integrated MPU-6050 yaw is subject to drift.

# 8. Debugging and Performance Progress

USB and serial connectivity were investigated using Linux kernel logs and serial-port diagnostics.

The Raspberry Pi showed intermittent undervoltage warnings in dmesg; this is important because USB devices and the scanner can become unreliable during power instability.

A CP210x USB-serial timeout was also observed in the kernel log.

The scanner's GUI initially suffered from severe lag and crashes during point-cloud rendering.

py-spy profiling showed the main GUI thread spending significant time in VisPy MarkersVisual.set_data() / _upload_data(), identifying point-cloud GPU/CPU upload as an important performance area.

Rendering and camera behavior were subsequently improved; zoom/camera interaction was reported as working.

The scanner uses event-driven rendering and dirty-state concepts to avoid unnecessary repeated rendering.

LiDAR acquisition, autosave and ESP32 communication are handled separately from the GUI thread.

# 9. Current Project Architecture

Data flow:
YDLIDAR X2 → Raspberry Pi 5 → Python scanner → NumPy 3D processing → VisPy/OpenGL → Real-time point cloud

Rotation/control flow:
Python scanner → USB Serial → ESP32 → DRV8825 → NEMA17 → LiDAR platform

Motion verification:
Platform movement → MPU-6050 → ESP32 → ROTATED / ROTATION_NOT_DETECTED status

# 10. Current Status

| Feature | Status |
| --- | --- |
| 2D LiDAR acquisition | Working |
| ESP32 serial communication | Working |
| Stepper motor control | Working |
| NEXT / MOVE / HOME protocol | Working |
| Logical platform angle reporting | Working |
| MPU-6050 integration | Working / being tuned |
| Physical rotation detection | Being tuned for small movements |
| Real-time 3D visualization | Working; performance optimized during development |
| Camera zoom/pan/rotation | Working |
| 3D mapping synchronization | Next integration/validation step |

# 11. Key Remaining Work

Validate the 1.5° movement configuration across a complete scan.

Tune MPU-6050 physical-rotation detection so small real movements are detected without false positives from vibration.

Ensure the Raspberry Pi scanner waits for the correct movement-completion signal before associating the next LiDAR slice with its platform angle.

Validate 3D point-cloud reconstruction over a complete 360° platform sweep.

Measure scan speed, point density, angular accuracy and repeatability.

Continue optimizing rendering if very large point clouds cause performance degradation.

Use the completed scanner as the base for future AI-enabled applications.

# 12. Project Summary

The project has progressed from a basic 2D LiDAR acquisition system to an integrated embedded 3D scanning platform. The Raspberry Pi 5 handles LiDAR acquisition, data processing and visualization, while the ESP32 provides deterministic stepper control and now incorporates the MPU-6050 as an additional physical-motion check. The existing serial control protocol has been preserved to maintain compatibility between the ESP32 firmware and Raspberry Pi scanner.

The main technical challenge addressed during development has been reliable synchronization between LiDAR acquisition, mechanical rotation and real-time visualization. The current work focuses on validating physical-rotation detection and completing reliable 3D reconstruction across a full platform sweep.

# 13. One-Line Project Explanation

A low-cost 2D-to-3D LiDAR system that mechanically rotates a YDLIDAR X2, synchronizes each scan with platform angle, reconstructs a 3D point cloud on a Raspberry Pi 5, and provides real-time visualization with an AI-ready architecture.